TRAIL ASSESSMENT FORM

| Doc. # | 1011 | Trail Name |  |  | Five Corners Path North | Trail Type: | Wide Trail - M |  | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Hillside Street – 1110 - 1120 – 1141 – 1140 – 1135 Border Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Snow 12/2 , Light rain 12/10 – Partial snow coverage up to 1 inch on trail. Currently sunny |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trail begins on the northside of Hillside Street at a granite marker labeled Five Corners Path. Trailhead is located to the right of a small parking area 1/10 of a mile west of Blue Hills River Road. - Trail begins as rocky 6’ wide path with bn. - Bn and sbd at 125 feet. 10” across trail except for 2’. - Unnamed path leaves on right at 330 feet. - Small unnamed path leaves on right at 450 feet. - Bn at .11 miles. Side and overhead. - Bn at .13 miles. - Bn at .15 miles. - Bn at .17 miles. Trail narrows to body width. - Half Way path crosses at intersection marker 1110 at .19 miles. - Bn at .10 -.20 miles. - South Skyline crosses at .28 miles. - Bn at .29 miles. Side and overhead. - Coon Hollow path crosses at intersection marker 1120 at .34 miles. - Green spot and yellow triangular blazes begin on the Five Corners Path. - Bn at .35 miles. - Bn at .48 miles. Overhead. - Bn at .50 miles. - North Skyline and Wolcott Path cross at intersection marker 1141 at .57 miles. - Bn at .57 miles. - Trail opens up very wide at .63 miles. - Dead conifer rotted on trail side on right at .65 miles. 14”. Remove. (1) - Hemenway Hill Path crosses at intersection marker 1140 at .81 miles. - Eustis trail leaves on right 10 yards after this intersection. - Rd at .84 miles. R to L. Dt on left. Clean. - Dt running off trail on right at .89 miles. Clean. - Rd at .9 miles. L to R across trail. Good dt on right. Clean. - Sbd at .93 miles on left and into trail. Cut back or remove. - Trail splits into two at its intersection with Border Path at intersection marker 1135 at .94 miles. Bartol path continues straight. - On the left fork just prior to the intersection with the Border Path there is a large hole in the treadway caused by erosion that needs to be filled. It may be helpful to install a wb uphill from this with adequate dt. (2) |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - (2) Large hole in the treadway caused by erosion that needs to be filled. It may be helpful to install a wb uphill from this with adequate dt. - Significant bn needed at multiple areas. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - (1) Dead conifer rotted on trail side on right at .65 miles. 14”. Remove. |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Hillside Street to Half Way Path marker 1110 - .19 miles. - Hillside Street to South Skyline intersection - .28 miles. - Hillside Street to Coon Hollow Path marker 1120 - .34 miles - Hillside Street to North Skyline and Wolcott Path intersection at marker 1141 - .57 miles. - Hillside Street to Hemenway Hill Path at intersection marker 1140 - .81 miles. - Hillside Street to intersection with Border Path at marker 1135 - .94 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones |  |  |  | Assessment date: |  | 12/11/2007 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)