TRAIL ASSESSMENT FORM

| Doc. # | 1014 | Trail Name |  |  | Hemenway Hill Path West (rev.1) | Trail Type: |  | Wide - M | # Condition |  | Fair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 1203 –1204 – 1180 - 1178 - 1140 - Rotch Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Warm - melting snow and ice – Snow cover on trail was 0-2 inches |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead leaves on the west side of the Chestnut Run Path at intersection marker 1203 at road width. - Unnamed trail leaves on left at .02 miles. - Small unnamed trail leaves on right taking the triangular yellow blazes with it at .04 miles - Additional spur of Hancock Hill Path leaves on left at intersection marker 1204 at .05 miles. - Hancock Hill Path leaves on left at .05 miles. - Rd running L to R at .07 miles. Dt on right. Needs cleaning. - Rd running L to R at .08 miles. Dt on right. Needs cleaning. - Wooden (log) wb running L to R at .14 miles. Dt on R and L. Needs cleaning and dig out.. - Three cv under road running L to R at .22 miles. Large dt on left feeding all inlets. 10”, 13”, 10’ plastic pipe. Well maintained rkw on both sides. Outlets and inlets require cleaning. Inlets full of leaves. - Breakneck Ledge Path crosses north/south at intersection marker 1180 at .24 miles. Hemenway Hill Path turns south here and shares the same treadway with the Breakneck Ledge Path for .03 miles. - Hemenway Hill Path leaves the Breakneck Ledge Path on the right (west) at intersection marker 1178 at .27 miles. - Cv under trail for stream at .29 miles. L to R. Large pool for overflow on R with Rkw. 24” dia. clay pipe. Outlet has sbd traversing stream and another sbd in 20 feet. Needs to be removed. (1) - Wb (dead tree 6”) crosses trail at .32 miles. L to R. Submerged. Dt on R and L. Cleaning or replacement of wb and dt required. - Sbd extends halfway into trail at .38 mile. Needs to be cut back more to open road to vehicle traffic. Minor. - Moderate to severe erosion as trail ascends from .39 to .41 Mlles. Erosion varies from 1- 4 feet deep. Treadway is lower than surrounding area. Recommend addition of wb at start of downhill or parallel dt in or outside treadway. Needs review. (2) - Bn from .42 to .52 miles. - Jeffries Trail South to Hemenway Hill leaves on the left at .52 miles. - Jeffries Trail Spur South to Hemenway Hill leaves on the left at .53 miles. - Jeffries Trail North leaves on the right at .53 miles. Unofficial marker noted on tree at this point labeled 1154 with defaced “no mountain bike” blaze below it. - Trail begins it’s decent after this intersection. - Rd at .55 miles. R to L. Dt on Left. Cleaning required. - Wb at .60 miles. R to L. Dt on Left. Wb rotted and should be replaced. Clean wb and dt. (3) - Wb at .65 miles. R to L. Rkw on right. Dt on Left. Clean wb and dt. Dt should be deeper on left. - Wb at .70 miles. L to R. Dt on right. Clean wb and dt. Erosion follows this wb so its effect should be reviewed. (4) - Rd at .73 miles. R to L. Clean rd deeper and dt. Minor erosion. - Wb at .75 miles. L to R. Dt on right. Rkw on left. Dig out wb and clean dt. - Five Corner Path crosses at intersection Marker 1140 at .76 miles. Eustis Trail enters on right. - Trail narrows to a footpath following the intersection. - Bn at .79 - .83 miles. - Hemenway Hill Path ends with its intersection with the Rotch Path at .87 miles. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Cv under trail for stream at .29 miles Outlet has sbd traversing stream and another sbd in 20 feet. Needs to be removed. (3) Wb at .60 miles. Wb rotted and should be replaced. (4) Wb at .70 miles. Erosion follows this wb so its effect should be reviewed. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | (2) Moderate to severe erosion as trail ascends from .39 to .41 miles. Erosion varies from 1- 4 feet deep. Treadway is lower than surrounding area. Recommend addition of wb at start of downhill or parallel dt in or outside treadway. Needs review. |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Chestnut Run Path 1203 to intersection marker 1204 - .05 miles. - Chestnut Run Path 1203 to Breakneck Ledge Path at intersection marker 1180 - .24 miles. - Chestnut Run Path 1203 to Hemenway Hill Path leaves Breakneck Ledge Path at intersection marker 1178 - .27 miles. - Chestnut Run Path 1203 to Jeffries Trail South leaves on the left a - .52 miles. - Chestnut Run Path 1203 to Jeffries Trail North leaves on the right at .53 miles. - Chestnut Run Path 1203 to Five Corner Path crosses at intersection Marker 1140 - .76 miles. - Chestnut Run Path 1203 to Hemenway Hill Path ends at intersection with the Rotch Path at .87 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones revised 04/03/08 |  |  | Assessment date: |  |  | 1/8/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)