TRAIL ASSESSMENT FORM

| Doc. # | 3007 | Trail Name |  |  | Laurel Trail | Trail Type: |  | M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From the southern boundary fence at rte 128 to Bouncing Brook Trail, just before mk3130 .62 mi total |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | 1” snow covering some portions of trail, some icy spots, no precip past 7 days |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Wide trail starts at the southern fence at rte 128 105 yd mk3134 int w/ Braintree Pass Path coming in on W side 143 yd lbd(1) mk3122 337 yd cv cl nf mk3121(2) Laurel Trail turns right and descends at this point towards the edge of a swampy hemlock area 355 yd int w/ the end of Hemlock Bound Foot Path coming in on SE side 510 yd du W side, trail follows swamp on E side 743 yd cv cl broken semi-fc(3) 753 yd mk3111 trail on N side cv cm nf(4) 756 yd possible cv under heavy debris investigate (there is water flowing N/S under trail at this point) 724 yd lbd(5) .61 mi shallow stream crossing 4’ wide (max) not really an obstacle, drains into the hemlock stand .61 mi 30” cv cl nf(6) .62 mi int with BBT no mk |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - trail saws, crew of 3 - mk missing (5) trail saws, crew of 3 (6) cv cleanout |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - cv may need rebuilding - cv may need rebuilding or replacement. It is above grade and could be considered a hazard |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | .62 mi from fence at 128 to BBT |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 01/23/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)