TRAIL ASSESSMENT FORM

| Doc. # | 3013 | Trail Name |  |  | Barberry Bush Spring Trail | Trail Type: |  | M | # Condition |  | Good* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | mk3121 off Laurel Trail, to mk 3090 at Braintree Pass Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Light snow ¼”-1” 33F. A few inches of rain has fallen last few days. |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | mk 3121 trail heads W off Laurel Trail, up rocky slope. Trail is wet, with water flowing, but no erosion due to all the rock. No visible source of water flow. Trail is mostly uphill, until 453 yd. sbd(1) 698 yd. end of wet area. 771 yd. Trail ends at mk3090 int with Braintree Pass Path |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | (1) *where trail levels off it becomes a swamp, associated with Wild Duck Pool on the S side, but here the trail is a huge puddle. May be able to help drain with some knicking on the N side, but still will have the water flowing across the trail seasonally at least. Recommend installing a bog bridge or some rkw. |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | 771 yd from mk 3121 at Laurel Trail to mk 3090 at Braintree Pass path |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 02/02/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)