TRAIL ASSESSMENT FORM

| Doc. # | 3024 | Trail Name |  |  | Branch Path | Trail Type: |  | M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From end of Streamside Edge Path(SEP) N to mk3066(3060?) at SEP |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Clear, no recent precip |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Trail begins at southern end of SEP 354yd possible cv under causeway no issues 384yd mk3071 Hemlock Bound Foot Path crosses 537yd some drainage issues with waterflow on trail recommend rd and/or ditch with armored water crossing at this point. Not a big issue 607yd mk3066 trail ends at SEP |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | Trail is in good shape except where noted |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | 607yd loop from S end of SEP, to mk3066 at SEP |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 03/10/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)