TRAIL ASSESSMENT FORM

| Doc. # | 2001 | Trail Name |  |  | Skyline Trail East Hillside Street to Route 28 | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Hillside Street – 2054 - 2117 – 2141 – 2165 – 2210 – Route 28 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | No Rain in last 24 hours. Clear and cold in 30’s. Some ice on trail |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead leaves at a granite marker from the south side of Hillside Street at the rear of a small parking turnout directly across the street from Reservation Headquarters. Headquarters is located .4 miles north of the Houghton’s Pond parking area. There is signage on a tree labeled Skyline East to Buck Hill. Trail is blazed with blue rectangles. - Cv under treadway at 150 feet for streambed running L to R. Rwk wells with 14” metal pipe. Rock in streambed on left blocking inlet and debris to be removed. - Additional cv for the same streambed is located immediately to the left under the Skyline Connector road. Rkw on L and R with 18” metal pipe. Needs cleaning as well. - Skyline Trail enters the Skyline Connector on the right at 240 feet and shares the same treadway for a short distance. - Cv under treadway at .15 miles for streambed running L to R. Rwk/mortar const. w/ 14” clay pipe visible on right. Cv is heavily impacted on L with Sbd and debris on L. No visible pipe showing on left. (1) - Skyline leaves the Skyline Connector on the left at intersection marker 2054 at .22 miles. - Bn at .23 miles. - Sbd crosses trail at .27 miles. Bow saw work. - Blazed tree down to left of trail at .28 mile. Relocate blazing. - Lbd crosses the treadway at .31 miles. Two large limbs cross trail requiring chainsaw removal. (2) - Trail begins ascending Tucker Hill at .35 miles. - Bn at .38 miles. - Tucker Path crosses at .44 miles. Tucker Path is blazed with greed dots with white triangular direction blazes pointing south. - Log wb at .39 miles providing large step up. Needs to be dug out and ditched. - Unnamed path crosses at .46 miles. - Minor erosion noted ascending Tucker Hill at .49 w/ Rkw step acting as a wb. R to L. Opportunities for additional wb in this area to reduce erosion. Erosion stops at .54 miles as treadway enters a rock slab area. - Bn at .62 miles. - Unnamed path crosses at .64 miles. Spur Path to Athol Path. - Athol Path enters on left at .67 miles crossing the summit of Tucker Hill. - Treadway begins a steep descent off Tucker Hill at .73 miles through an are of minor erosion as treadway splits into two paths to avoid an area of rock scrambling. - Treadway enters another area of minor erosion at .75 miles with 2-abandoned Rkw wbs running L to R. Need to be dug out and dt on right. - Blazed lbd on left at .8 miles. Needs to be cut back as it extends into the treadway and blaze needs to be relocated. 18”dia, chainsaw work. - Dark Hollow Path crosses at .82 miles and uses the same treadway for approximately 50 yards. Trail is blazed with green dots with white triangular blazes pointing north. - Skyline leaves the Dark Hollow Path on the right at .83 miles. - Bn at .84 miles with downed tree limbs. Bow saw and lopper work. - Treadway enters an area of moderate runoff at .84 miles. Possible bog bridge location. (3) (Photo attached SLE01). - Treadway enters an area of moderate runoff at .87 miles. Possible bog bridge location. (4) - Bn at .88 miles. - Bn at .92 miles. Additional blazes in this direction would be helpful here as route finding becomes challenging. (5) - Lbd crosses trail at .93 miles. Old dead tree needs removal. Chainsaw work. - Trail begins ascending N. Boyce Hill at .94 miles. - Unnamed path crosses at intersection marker 2117 at 1.0 miles. Blazed with white triangles pointing south. Blue rectangular blaze labeled KCE with design noted as trail reenters footpath. - Treadway passes Whale Rock on right at 1.13 miles. - Deer Hollow Path crosses at intersection marker 2141 at 1.17 miles. - Bn at 1.18 miles. - Trail begins ascending Buck Hill at 1.25 miles with bn. - Trail enters an area of minor erosion at 1.27 miles. Rkw steps extend from here through 1.28 however 2 additional Rkw wb and dt running L to R above the steps would improve the erosion issue in this area. (6) - Bn at 1.33 miles. - Abandoned rwb at 1.37 miles needs to be dug out and dt. - Unnamed path crosses at 1.39 miles. - Treadway crosses the summit of Buck Hill at intersection marker 2165 at 1.41 miles. - Unnamed path leaves on the left at 1.42 miles. - Large cairn to left of treadway as the crossing the Buck Hill summit continues at 1.44 miles. - Trail begins the descent of Buck Hill at 1.45 miles. - Unnamed path leaves on the right at 1.45 miles. - Rkw wb at 1.46 miles w/ bn. L to R. (New) - Trail descends through an area of minor erosion however what appears to be new wbs in this area are addressing. - Rkw wb at 1.47 miles w/ bn. L to R. (New) - Rkw wb at 1.48 miles w/ bn. L to R. (New) Needs better dt on right. - Rkw steps begin at 1.49 and continue through 1.52 miles with nice retaining walls. Loose rock needs to be removed. - Buck Hill Path crosses at 1.52 miles. - Unnamed path crosses at intersection marker 2210 at 1.63 miles. (May be an extension of Buck Hill Path. - Forest Run Path crosses at 1.68 miles w/ white triangular blazes pointing north. - Cv under treadway beside route 28 at 1.7 miles. L to R. Severely impacted. 20 dia. pipe ½ full of debris on right and ¾ full on left with water in dt backing up. This looks like a big job to me as the dt extends 50 feet uphill. I’d consider this state highway work. (7) - Skyline Trail crosses Route 28 at 1.7 miles at a granite marker labeled Skyline Trail 4.7 miles and a DCR Map board. Turnoff parking available for approximately 3 vehicles. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Cv under treadway at .15 miles. Cv is heavily impacted on L with Sbd and debris on L. No visible pipe showing on left. - Lbd crosses the treadway at .31 miles. Two large limbs cross trail requiring chainsaw removal. - Treadway enters an area of moderate runoff at .84 miles. Possible bog bridge location. (Photo attached SLE01). - Treadway enters an area of moderate runoff at .87 miles. Possible bog bridge location. - Bn at .92 miles. Additional blazes in this direction would be helpful here as route finding becomes challenging. - Trail enters an area of minor erosion at 1.27 miles. Rkw steps extend from here through 1.28 however 2 additional Rkw wb and dt running L to R above the steps would improve the erosion issue in this area. - Cv under treadway beside route 28 at 1.7 miles. Severely impacted. Pipe is ½ full of debris on right and ¾ full on left with water in dt backing up. This looks like a big job to me as the dt extends 50 feet uphill. I’d consider this state highway work. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None Noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Hillside Street to Skyline leaves the Skyline Connector on the left at intersection marker 2054. - .22 miles. - Hillside Street to Tucker Path. - .44 miles. - Hillside Street to Athol Path and the summit of Tucker Hill. - .67 miles - Hillside Street to Dark Hollow Path - .82 miles - Hillside Street to intersection marker 2117. - 1.0 miles - Hillside Street to Whale Rock. - 1.13 miles - Hillside Street to Deer Hollow Path at intersection marker 2141. - 1.17 miles - Hillside Street to summit of Buck Hill at intersection marker 2165. - 1.41 miles. - Hillside Street to Buck Hill Path. - 1.52 miles - Hillside Street to intersection marker 2210. - 1.63 miles. - Hillside Street to Forest Run Path. - 1.68 miles - Hillside Street to Route 28. - 1.7 miles |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones |  |  | Assessment date: |  |  | 2/17/2008 |  |

SLE01

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)