TRAIL ASSESSMENT FORM

| Doc. # | 2013 | Trail Name |  |  | Angle Path | Trail Type: |  | M(MTB) | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From mk2075 at Bugbee Path, heading generally south to mk2100 at Old Rte 128 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Clear and unseasonably dry |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Wide trail starts at mk 2075 on the south side of Bugbee Path 521yd mk2092 int with Burnt Hill Path 598yd (2095)narrow path(Bolton Foot Path) on left 750yd mk2100 no mk trail ends at Old Rte 128(1)(2) |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Install mk |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | (2)Where the trail ends at Old Rte 128, the last 10yds descend sharply and the tread is washing out onto the pavement. Investigate ways to alleviate this, possibly with rockwork and/or re-routing |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | from mk2075 521yd to mk2092 750yd to mk2100 trail end at Old Rte 128 |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 04/18/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)

MTB(mountainbikes allowed)

(mk2042) meaning not on DCR map