TRAIL ASSESSMENT FORM

| Doc. # | 2030 | Trail Name |  |  | Dark Hollow Path | Trail Type: |  | M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Chickatawbut Rd to 2112 – 2094 – 2096 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Warm (mid 30s), dry last few days, bare ground |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - The gated trailhead is located on Chickatawbut Rd. The trail is larger than a primitive trail but barely passable for a vehicle. - Headquarters Path crosses at marker 2112 at 10 feet. - Wet spot at 346 feet that looks like someone cut the downslope bank to allow water to flow off the trail. The area could benefit from a modest swale to collect the water and move it off the trail. - Intersection with Skyline and Red Dot Trail at .24 miles. - Green Dot enters at R at .29 miles and continues to the end of the trail. - Double cv at .31 miles. Pipes are 8” steel cv. Need to clean inflow and improve outflow to enhance flow. - Red Dot crosses at .49 miles at marker 2094. - Trail ends at Bugbee Path at .60 miles. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Trail could benefit with the weed whacker first 200 feet to clear low new brush. - Should consider replacing double cv to improve waterflow. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Headquarters Path at 10 feet. - Skyline Trail and Red Dot at .24 miles - Green Dot intersects and joins at .29 miles. - Red Dot crosses at .49 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Bob Flagg & Robert Shine |  |  | Assessment date: |  |  | 2/20/12 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)

V5 NDR