TRAIL ASSESSMENT FORM

| Doc. # | 2036 | Trail Name |  |  | Ayer Trail North | Trail Type: |  | Wide – M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 2094 – Skyline Trail – Tucker Hill Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Sunny in the 40’s. – No precipitation last 48 hours |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead is located on the west side of the Dark Hollow Path at intersection marker 2094. - Although the location of this trail is shown on the latest (9/09) DCR map, it is still shown as an unnamed trail. Future maps will show this as the Ayer Trail. - Trail leaves intersection marker 2094 headed northwest ascending at road width. - Overhead bn at .01 miles - Overhead bn at .06 miles - Overhead bn at .07 miles - Unnamed path crosses the Ayer Trail at .10 miles at the height of land. - Treadway begins descending. - Side bn at .2 miles to keep treadway at road width if desired. - Skyline Trail crosses the Ayer Trail at .21 miles. - Overhead bn at .22 through .24 miles. - The Ayer Trail ends with its intersection with the Tucker Hill Path at .24 miles |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Trailhead at 2094 to Skyline Trail - .21 miles - Trailhead at 2094 to Tucker Hill Path - .24 miles |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 11/22/2009 |  |  | Assessment date: |  |  | 11/17/2009 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)