TRAIL ASSESSMENT FORM

| Doc. # | 6011 | Trail Name |  |  | Cosmos Path West | Trail Type: |  | Wide and Narrow- M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 138 Parking Area – 6830 - K Path - 6700 – 6850 – 6800 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Moderate rain in last 24 hours – Sunny and 32 degrees |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead is located on the west side of the route 138 parking area located 5/8 of a mile north of route 93/128 at exit 2. - Trail leaves from an opening in the parking area fence at the DCR map board at footpath width and shares the same treadway as the Green Dot Loop Trail. - Moderate to severe erosion leaving the trailhead for 25 feet. Most likely caused by runoff from the parking area. (1) - No blazes are noted for the first 220 feet of treadway. Makes route finding difficult due to the number of spur paths in the area. (2) - Unnamed path leaves on left at 100 feet. - Unnamed path leaves on right at 130 feet. - Standing water in the treadway from 170 feet through .11 miles. Runoff makes using treadway impractical causing users to bushwhack around. Possible trail relocation or bog bridge location. (3) - Unnamed path leaves on left at 250 feet. - White temporary square blazes noting trail at 270 feet. - Treadway opens up to road width at .11 miles. - Unnamed path leaves on the right at road width at .19 miles. Bootleg intersection marker noted on tree labeled 6877. Cosmos Path/Green Dot Loop Trail continues to the left. - Lbd crosses the treadway at .25 miles. Can be stepped over. - Treadway begins ascending at .29 miles going around Little Blue Hill. - Treadway intersects Moulton Path at intersection marker 6830 at .31 miles. Cosmos Path leaves on the right (NW) at this intersection descending slightly at road width. - Treadway levels out at .39 miles. - Lbd extends into the treadway at .46 miles. Can be pulled off. - Cosmos Path enters K Path briefly at .47 miles however immediately leaves on the left (west) at .48 miles at path width. - Treadway enters a meadow for equestrian events at .49 miles - Sbd crosses the treadway at .51 miles. Can be stepped over. - Unnamed path leaves on the right at .56 miles to Green Street at road width. - Bd crosses the treadway at .62 miles. Can be carried off. - Unnamed path leaves on the right at .63 miles to Green Street as Cosmos Path parallels Green Street at footpath width. - Two lbd extend into the treadway at .65 miles. Can be walked around. - Unnamed path leaves on the right at road width at .85 miles into Laurel Ridge. - Cosmos Path leaves the wide road on the left (south) at .86 miles at footpath width. Wide road continues into Laurel Ridge residences. - Unnamed path leaves on the right at .94 miles. - Cosmos Path crosses the Skyline Trail at intersection marker 6700 at .97 miles - Unnamed path leaves on the right at .99 miles. - Unnamed path leaves on the right to Royal Street at 1.1 miles. The Royal Street entrance is marked with 3 boulders. - Cosmos Path enters K Path at intersection marker 6850 at 1.22 miles. At this point Cosmos Path turns left (north) and shares the same treadway as the K Path at road width. - Cosmos Path enters an intersection with the Skyline Trail and ends at intersection marker 6800 at 1.32 miles. At this intersection, K Path continues straight, the Skyline trail crosses while the Green Dot Loop trail turns right (North) towards Little Blue Hill. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Moderate to severe erosion leaving the trailhead for 25 feet. Most likely caused by runoff from the parking area. (1) - No blazes are noted for the fist 220 feet of treadway. Makes route finding difficult due to the number of spur paths in the area. (2) - Standing water in the treadway from 170 feet through .11 miles. Runoff makes using treadway impractical causing users to bushwhack around Possible trail relocation or bog bridge location. (3) |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None Noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Trailhead to Moulton Path at intersection marker 6830 - .31 miles - Trailhead to K Path - .47 miles - Trailhead to Skyline Trail at intersection marker 6700 - .97 miles - Trailhead to K Path at intersection marker 6850 -.1.22 miles - Trailhead to Cosmos Path ends at intersection marker 6800 - 1.32 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 12/29/2008 |  |  | Assessment date: |  |  | 12/18/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)