TRAIL ASSESSMENT FORM

| Doc. # | 5002 | Trail Name |  |  | Swamp Trail South FBH Map | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 5365 – 5364 – 5361 – 5332 - Pipeline |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Rain last 24 hours, Cloudy in the 30’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead is located on the east side of Ponkapoag Pond and shares a portion of the same treadway used by the Green Dot Loop Trail. It leaves from intersection marker 5365 heading south following the green dot blazes. The Duck Pond Foot Path (FBH map) also leaves on the left (east) at road width from this intersection. - This trail although not named on the DCR map gets its name from the FBH map. - Wooden wb crosses treadway at .02 miles. L to R. Dig out and dt on right. - Treadway enters an intersection at intersection marker 5364 at .04 miles at the entry to the AMC Camps. There is a pump house to the left of treadway and an unnamed footpath leaving on the right to Ponkapoag Pond and cabins. - Wooden wb crosses treadway at .06 miles. R to L. Dig out and dt on left. - Unnamed path leaves on right at .07 into AMC Camps. - Three wooden wb cross treadway at .12 miles. R to L. Dig out and dt on left. - Treadway enters a road width path on the right at .14 miles at the height of land into the AMC Camps and cabin area. - Ponkapoag Path crosses at road width at intersection marker 5361 at .19 miles while multiple paths continue leave on right to cabins. - AMC Caretaker driveway leaves on right at intersection marker 5332 at .24 miles. - Unnamed path leaves on right at .31 miles at I beam and wooden wb w/ rkw crossing the treadway. L to R. Dig out and clean dt. - Wooden wb w/rkw crossing the treadway at .39 miles. L to R. Dig out and clean dt. - Cv under the treadway at .42 miles. 36” cement pipe in streambed. - The Swamp Trail (FBH Map) ends with its intersection with the pipeline at .49 miles. At this intersection the Acton Path begins while continuing to follow the green blazes on the south side of Ponkapoag Pond. An unnamed footpath continues south to the reservation boundary. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Cv and dt cleaning as noted in assessment. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Intersection marker 5365 to AMC Camps at intersection marker 5364 - .04 miles - Intersection marker 5365 to Ponkapoag Path at intersection marker 5361 - .19 miles - Intersection marker 5365 to AMC Caretaker at intersection marker 5332 - .24 miles. - Intersection marker 5365 to The Swamp Trail (FBH Map) ends with its intersection with the pipeline at .49 miles |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 4/8/08 |  |  | Assessment date: |  |  | 4/7/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)