TRAIL ASSESSMENT FORM

| Doc. # | 5011 | Trail Name |  |  | Fire Trail South | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 5365 – 5381 - 5384 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Light rain last 24 hours, Party Sunny in the 40’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead is located on the left side (east) of the Green Dot Loop Trail (Swamp Trail FBH map) and leaves southeast from intersection marker 5365 at road width and ascending. - Downed limbs with Bn at 60 feet. Bow saw. - Wooden wb crosses at 130 feet. R to L. Rotted and ineffective. Needs to be replaces and dt on left. (1) - Treadway reaches a height of land at 150 feet. - Treadway enters an intersection at intersection marker 5381 at .16 miles. An unnamed link path leaves on the left (NE) to the Three Pines Path as the Fire Trail continues to the right heading southeast. - Treadway narrows somewhat with bn and debris removal at .17 miles. - Treadway reopens to road width at .19 miles. - Bn from .22 through .23 miles. - The Fire Trail ends with its intersection with Hemlock Road at intersection marker 5384 at .26 miles. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Wooden wb crosses at 130 feet. Rotted and ineffective. Needs to be replaces and dt on left. (1) |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Trailhead to Treadway enters an intersection at intersection marker 5381 at .16 miles. - Trailhead to the Fire Trail ends at its intersection with Hemlock Road at intersection marker 5384 at .26 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 4/15/08 |  |  | Assessment date: |  |  | 4/14/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)