TRAIL ASSESSMENT FORM

| Doc. # | 4010 | Trail Name |  |  | Crags Foot Path East 4091 to 4155 | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 4091 – 4095 – 4142 – 4145 – 4151 – 4153 - Skyline Trail - 4155 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Rain in last 48 hours, Sunny in the 30’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - For the purpose of this trail description, Crags Foot Path West will be surveyed using both the DCR and FBH maps to capture it in its entirety. There are some sections of this trail that cover unnamed paths on the DCR map. The old FBH maps refer to this trail as the Crags Foot Trail. - Trailhead leaves from the Indian Camp Path at intersection marker 4091. - Bn at .01 miles. - Green Dot Loop Trail enters on the right at intersection marker 4095 at .09 miles. Crags Foot Path and the Green Dot Loop will share the same treadway until noted otherwise. Green dot blazes identify the trail. - Bn from .12 through .32 miles. Small trees extending into the treadway. - Downed limb at .18 miles with bn. 8’. Bow saw and loppers. - Short Path leaves on left at .21 miles. Old intersection marker 4124H is noted at this location. Old FBH maps note this as well. - Cv under treadway in streambed at .42 miles. R to L. Rkw/earthen construction. 30” pipe. Flowing well with minor cleaning required. Flowing water is Furnace Brook. - Unnamed path leaves on right at intersection marker 4142 at .46 miles. Road width connector to Indian Camp Path. Old FBH maps show this as the Furnace Brook Trail. - Runoff in treadway at .51 miles. Treadway should have a rkw dt with appropriate dt into woods installed to resolve. (1) - Two cvs located at .53 miles with remains of a third one. The first cv is 6” clay pipe with significant blockage on upstream side and half blocked on downstream side. Evidence of another cv but upstream side is completely buried and downstream side is under rock overhang. Last cv is 10” clay pipe that is functioning pretty well. - Old Furnace Brook Path leaves on left at intersection marker 4145 at .55 miles. - Bn at .61 miles - Crags Foot Path leaves on the right at intersection marker 4151 at .64 miles at road width. The Green Dot Trail / Quarry Path (trailhead FBH) continue straight and share the same treadway. - Unnamed path enters on the left at .68 miles. - Bd to left of treadway at .7 miles. Pull off treadway. - Trail enters an intersection at intersection marker 4153 at .71 miles as the Crag Trail leaves on the right and Crags Foot Path bears left (southeast) (FBH). - Erosion noted in the treadway at this intersection and along the treadway to .75 miles. Needs dt on left of treadway as well as two wb to resolve.(2) - Rkw wb crosses R to L at .76 miles. Ineffective. Needs to be dug out and dt. - Skyline Trail enters on the left at .78 miles and shares the same treadway. - Crags Foot Path ends with its intersection with the pipeline at intersection marker 4155 at .79 miles. Signage on tree denotes Green Dot and Skyline East to St. Moritz and Skyline West to Great Blue. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Runoff in treadway at .51 miles. Treadway should have a rkw dt with appropriate dt into woods installed to resolve. (1) - Erosion noted in the treadway at this intersection and along the treadway to .75 miles. Needs dt on left of treadway as well as two wb to resolve.(2) |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Trailhead intersection marker 4091 to Green Dot Loop Trail enters at intersection marker 4095 - .09 miles. - Trailhead intersection marker 4091 to Short Path leaves on left - .21 miles. - Trailhead intersection marker 4091 to intersection marker 4142 - .46 miles. Unnamed path (Furnace Brook Trail) - Trailhead intersection marker 4091 to Old Furnace Brook Path at intersection marker 4145 - .55 miles. - Trailhead intersection marker 4091 to Crags Foot Path leaves on the right at intersection marker 4151 - .64 miles. - Trailhead intersection marker 4091 to intersection marker 4153 - .71 miles. Crag Trail (FBH) and Crags Foot Path. - Trailhead intersection marker 4091 to Skyline Trail - .78 miles - Trailhead intersection marker 4091 to Crags Foot Path ends at intersection marker 4155 - .79 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones |  |  | Assessment date: |  |  | 3/13/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)