TRAIL ASSESSMENT FORM

| Doc. # | 1007 | Trail Name |  |  | Houghton Path North | Trail Type: | Wide Trail - M |  | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Hillside Street - 1143 - Wolcott Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Snow 12/2 , Cloudy and cold – Partial snow coverage up to 1 inch on trail |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trail leaves from the granite marker on the north side of Hillside Street 1/8 mile west of the Houghton Pond parking area. - Unnamed spur trail leaves on the left to Hillside Street at 70 feet. - Wide road with minor brushing on roadside needed - Unnamed trail crosses at 285 feet. Yellow triangular blazed. - Rd at 500 feet. L to R. Dt on left. Removes water from trail. Dt needs cleaning. - Wooden wb at .13 miles. R to L. Needs to be dug out on uphill side and dt needs to be better defined on left. Removes water from trail. (1) - Half Way Path enters on left at intersection marker 1143 at .22 miles. - South Skyline crosses at .24 miles. - Minor brushing on roadside needed at .4 miles - Houghton Path Ends with it’s intersection with Wolcott Path at .49 miles |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | (1) Wooden wb at .13 needs to be dug out on uphill side and dt needs to be better defined on left. Removes water from trail. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | None Noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Hillside Street to Half Way Path intersection and marker 1143 -.22 miles. - Hillside Street to South Skyline intersection - .24 miles. - Hillside Street to end and intersection with Wolcott Path - .49 miles |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones |  |  |  | Assessment date: |  | 12/7/2007 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)