TRAIL ASSESSMENT FORM

| Doc. # | 2004 | Trail Name |  |  | Buck Hill Path | Trail Type: |  | M(MTB) | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From mk2164 at Headquarters Path, generally SE following the NE contours of Buck Hill, to mk2183 at Buck Hill Trail |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Clear, no recent precip |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Trail begins at Hillside St with fire gate and granite marker and ‘no motorized vehicle’ placard 10yd mk2164 Headquarters Path crosses 169yd nf wood wb( see ‘minor problem areas’ below) 189yd nf wood wb(“) 319yd nf wood wb(“) 329yd nf wood wb(“) 339yd nf wood wb (“) 354 ydmk2182 AMC Foot Path crosses 374yd nf wood wb (“) 436yd large nf wb(“) 492yd el.447 apex of trail 607yd no mk Skyline Trail crosses 627yd nf wood wb remove 682 yd older nf wb recommend rd(“) The trail after this point has been covered with gravel which is washing downtrail. See (“) 774yd mk2183 trail ends at Buck Hill Trail |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | (“)No major drainage/erosion issues, but need to address the nf wbs . To avoid future erosion patterns, removal of all of the wbs and the installing a few well-placed rolling grade dips is recommended. This would be an easy place to get heavy equipment in to do the work. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | 354yd to AMC Foot Path 607yd to Skyline 774yd to Buck Hill Trail |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 03/24/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)

MTB(mountainbikes allowed)