TRAIL ASSESSMENT FORM

| Doc. # | 2006 | Trail Name |  |  | Fells Path | Trail Type: |  | M(MTB) | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From mk2152 at Buck Hill Trail, heading generally S, to mk 2140 at Bugbee Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Clear, no recent precip |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Wide trail begins at mk2152, Buck Hill Trail. Perenially wet area. 10yd cv empies onto the trail on L(1) 150yd du 302yd mk2149 int w/ Pig Rock Path. Trail bears right. 474yd mk2140 trail ends at Bugbee Path |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | (1) possibly rkw to alleviate muddy section |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | 474yd from mk2152 to mk2140 |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 03/24/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)

MTB(mountainbikes allowed)

(mk2042) meaning not on DCR map