TRAIL ASSESSMENT FORM

| Doc. # | 2008 | Trail Name |  |  | Bugbee Path | Trail Type: |  | M(MTB) | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | From the fire gate at Hillside street across from BH HQ and the stables, up the paved road on Breeze Hill, traveling generally east to mk2190 at int w/Forest Path |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Clear and dry |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | Wide Trail begins at (mk2062) at the fire gate at Hillside street across from BH HQ and the stables. Trail at this point is fresh gravel. At the gate there are some loose rocks that were an apparent attempt at defining the trailhead. White arrows indicating the MTB loop, Blue rectangles indicating Skyline. 15yd Lg clay cv stone-armored, diagonally L to R Old turtle fencing L needs removed 50yd Skyline enters with blue rect., white MTB arrow straight. 94yd f cv needs cleanout(1) 186yd mk2052 Skyline exits left 221 yd mk2054 trail becomes pavement and bears left up Breeze Hill. 529yd paved lot used for gravel, etc. (Prescott Foot Path) begins on R 677yd mk2070 int w/Tucker Hill Path. Bugbee becomes gravel again, paved road bears right. Green Dot joins Bugbee. White arrows. 743yd Lg cp cv w/dt nf nn 836yd hazardous nf wb(2) 887yd mk2075 Angle Path begins on right. Bugbee continues east with green dots and white arrows .66mi. rd into a dt L .74mi mk2096 int w/Dark Hollow. Bugbee turns right into a seasonally wet area, with a drainage knick on L. Short ‘connector’ path on L 10yd after mk2096 White arrows follow Bugbee, Green Dots turn left onto Dark Hollow .90mi mk 2111 Burnt Hill path begins on R. Evidence of fires in 2007 on L .94mi rd possibly an old wb. Possible cv also investigate .99mi mk2129 nf cv nn Doe Hollow Path crosses. White arrows turn right onto Doe Hollow. 1.03mi 2 f 12” cl cv dt following that need cleaning out(3) 1.07 mk2135 Beech Hollow Path begins on R White arrowed loop enters Bugbee again 1.10 mk2140 Fells enters R 2 nf cv, 1 more cv just after int, also(4) 1.14 mi (Farm Spring Foot Path)crosses 1.26mi mk2155 Oblique Path begins R dt on L needs cleaning(5) White arrows turn R on Oblique 1.28mi old wb nn remove 1.30mi dry cv 1.35mi cv with dt before and after(6) 1.37mi unofficial trail on R, possibly an old maintenance area 1.39mi Large lt 1.40mi cv nf nn 1.42mi cv nf nn 1.45mi mk2172 Pig Rock path enters L(7) 1.52mi mk2190 end of Bugbee Path at Forest Path crossing, although the trail is continuous. White arrows rejoin at this intersection. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - cleanout/investigate cv whether needed - remove wb - cleanout dt - cleanout cv - cleanout dt - cleanout dt and cv - mk is unreadable, replace |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  |  |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | From gate at Hillside st, to: Mk2070 677yd Mk2075 887yd Mk 2096 .74mi Mk2140 1.1mi Mk2155 1.26mi Mk2190 1.52mi end of Bugbee |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Steve Cobble |  |  | Assessment date: |  |  | 04/18/08 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd), ditches (dt)                      corrugated plastic(cp)

excessive erosion (ee)      leaning tree (lt))

intersection marker(mk)    dugout(du)

bog bridges (bb)             culvert replacement (cr)

non-functioning(nf)         functioning(fc)

damage(dg)

not needed(nn)

cm ( corrugated metal)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

clay(cl)

MTB(mountainbikes allowed)

(mk2042) meaning not on DCR map