TRAIL ASSESSMENT FORM

| Doc. # | 5002 | Trail Name |  |  | Swamp Trail South FBH Map Rev.1 | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | 5371 - 5365 – 5364 – 5361 – 5332 - Pipeline |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Rain last 24 hours, Cloudy in the 30’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead leaves from the east side of the Ponkapoag Trail at intersection marker 5371. Leaving the trailhead it initially heads east as treadway begins descending at road width. - Debris in treadway to be removed at 100 feet. - Duck Pond Foot Path (FBH map) leaves on the left (east) at 360 feet. - Debris in treadway to be removed at .1 miles. - Depression in treadway with runoff at .11 miles. Treadway would benefit if ditched on the right to drain runoff. (1) - Depression in treadway with runoff at .14 miles. Treadway would benefit if ditched on the right to drain runoff. (1) - Depression in treadway with runoff at .19 miles. Treadway would benefit if ditched on the right to drain runoff. - Treadway begins ascending at .24 miles. (1) - Green Dot Loop Trail enters on the right sharing the treadway with the Prescott Foot Path (FBH map) at .26 miles. Prescott Foot Path ends at this point. The Swamp Trail now shares the same treadway as the Green Dot Loop Trail. - Treadway enters an intersection at intersection marker 5365 at .29 miles. At this intersection an unnamed path (Fire Trail, FBH map) leaves on the left (southwest) at road width as the Swamp Trail continues south sharing the same treadway as the Green Dot Loop Trail. - Wooden wb crosses treadway at .31 miles. L to R. Dig out and dt on right. - Treadway enters an intersection at intersection marker 5364 at .33 miles at the entry to the AMC Camps. There is a pump house to the left of treadway and an unnamed footpath leaving on the right to Ponkapoag Pond and cabins. - Wooden wb crosses treadway at .35 miles. R to L. Dig out and dt on left. - Unnamed path leaves on right at .36 into AMC Camps. - Three wooden wb cross treadway at .41 miles. R to L. Dig out and dt on left. - Treadway enters a road width path on the right at .43 miles at the height of land into the AMC Camps and cabin area. - Ponkapoag Path crosses at road width at intersection marker 5361 at .48 miles while multiple paths continue leave on right to cabins. - AMC Caretaker driveway leaves on right at intersection marker 5332 at .53 miles. - Unnamed path leaves on right at .59 miles at I beam and wooden wb w/ rkw crossing the treadway. L to R. Dig out and clean dt. - Wooden wb w/rkw crossing the treadway at .68 miles. L to R. Dig out and clean dt. - Cv under the treadway at .71 miles. 36” cement pipe in streambed. - The Swamp Trail (FBH Map) ends with its intersection with the pipeline at .78 miles. At this intersection the Acton Path begins while continuing to follow the green blazes on the south side of Ponkapoag Pond. An unnamed footpath continues south to the reservation boundary. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Depression in treadway with runoff at .11, .14 and .19 miles. Treadway would benefit if ditched on the right to drain runoff. (1) |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Intersection marker 5371 to Duck Pond Foot Path (FBH map) leaves on the left (east) - 360 feet. - Intersection marker 5371 to Green Dot Loop Trail enters on the right -.26 miles. - Intersection marker 5371 to intersection marker 5365 - .29 miles. - Intersection marker 5371 to intersection marker 5364 - .33 miles. - Intersection marker 5371 to Ponkapoag Path crosses at intersection marker 5361 - .48 miles. - Intersection marker 5371 to intersection marker 5332 - .53 miles. - Intersection marker 5371 to The Swamp Trail (FBH Map) ends with its intersection with the pipeline. - .78 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 4/16/08 Rev.1 |  |  | Assessment date: |  |  | 4/7/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)