TRAIL ASSESSMENT FORM

| Doc. # | 5015 | Trail Name |  |  | Meadow Road North | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Randolph Street to Pipeline |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | Dry last 24 hours, Party Sunny in the 40’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Trailhead is located on the north side of Randolph Street just west of the Donovan School. Trail leaves at road width from a fire gate and granite marker labeled Meadow Road. - Unnamed path leaves on the right at 95 feet with bn. Spur path to the Donovan School parking area. - Treadway narrows at .1 miles with bn as trail descends. - Bn at .16 miles. Debris in treadway. - Unnamed path leaves on left at path width at .21 miles as treadway begins to descend. - Bn at .23 miles. Debris in treadway. - Cut logs in treadway at .25 miles as an unnamed path leaves on the left at road width. Need removal. - Bn at .27 miles. Overhead and side. Saplings extending into treadway. - Bn at .29 miles. Downed limbs in treadway with side brushing required. - Harmony Hill Farm entry gate is to the right of the treadway at.31 miles. - Side bn at .32 miles. - Unnamed path enters on the left at road width at .33 miles. - Treadway enters an intersection at intersection marker 5414 at .36 miles. Pasture Road leaves on the right at road width as Meadow Road continues northeast. - Treadway enters a section of asphalt paving at .40 miles. - Unnamed path leaves on right at path width at .44 miles. - Meadow Road ends with its intersection of the pipeline at .42 miles. - Paved treadway continues west to the horse bridge over route 24 and on to intersection marker 5401 at .68 miles |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Brushing as noted in the assessment. |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Trailhead on Randolph St. to intersection marker 5414 - .36 miles. - Trailhead on Randolph St. to Meadow Road ends with its intersection of the pipeline - .42 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 4/21/08 |  |  | Assessment date: |  |  | 4/21/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)