TRAIL ASSESSMENT FORM

| Doc. # | 4014 | Trail Name |  |  | Ridge Path North | Trail Type: |  | Wide - M | # Condition |  | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Chickatawbut Road – 4072 - 4047 |  |  |  |  |  |  |  |  |
| Recent Weather: |  |  | No rain last 48 hours, Sunny in the 30’s |  |  |  |  |  |  |  |  |
| TRAIL LANDMARKS |  |  | - Ridge Path leaves from the north side of Chickatawbut Road at a fire gate and granite marker labeled Ridge Path. It is located .52 miles from the point where the Skyline crosses Chickatawbut Road. No FBH intersection marker 4081 noted. - Unnamed Path leaves on the right at 38 feet. Connector path to Pine Tree Brook Path. - Treadway is road width ascending hill in good condition. - Bn at 75 feet. Debris in treadway. - Overhead bn at 90 feet. Small tree over treadway. - Height of land reached at 300 feet. - Unnamed path leaves on right at 390 feet. Road width path. - Bn at 500 feet to bring treadway back to road width. - Unnamed path (Great Dome Foot Path, old FBH map) crosses at intersection marker 4072 at .12 miles. - Treadway descending with bn at .13 miles. Pine boughs into treadway. - Two bd cross treadway at .16 miles. Cut back or pull off. 4’ and 8”. - Bn at .19 miles. Side bn to maintain road width. - Bd crosses treadway at .22 miles. - Lbd crosses treadway at .23 miles. Chain sawed already however additional required. - Bn from .24 through .25. Side and debris removal. - Two bd crosses treadway at .34 miles. Chain sawed already however additional required. Significant bn. - Treadway enters a 4-way intersection at .4 miles. Intersection not noted on FBH or DCR maps. Intersection created from a not shown, road width connector from Ridge Path to Sassaman Path. Ridge Path enters on right and continues on right while Sassaman Path enters on left and continues left. This intersection should have appropriate signage installed to eliminate confusion. (1) - Sbd at .42 miles. 2 people to remove. - Bn at .44 miles w/bd. 2 people to remove. - Ridge Path ends at intersection marker 4047 at .46 miles as it joins Sassaman Path on the left. |  |  |  |  |  |  |  |  |
| MINOR PROBLEM AREAS |  |  | - Treadway enters a 4-way intersection at .4 miles. Intersection not noted on FBH or DCR maps. Intersection created from a not shown, road width connector from Ridge Path to Sassaman Path. Ridge Path enters on right and continues on right while Sassaman Path enters on left and continues left. This intersection should have appropriate signage installed to eliminate confusion. (1) |  |  |  |  |  |  |  |  |
| MAJOR PROBLEM AREAS |  |  | - None Noted |  |  |  |  |  |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS |  |  | - Chickatawbut Road to unnamed path (Great Dome Foot Path, old FBH map) crosses at intersection marker 4072 - .12 miles. - Chickatawbut Road to 4 way intersection - .4 miles. Ridge Path enters on right and continues on right. - Chickatawbut Road to Ridge Path ends at intersection marker 4047 - .46 miles. |  |  |  |  |  |  |  |  |
| Report prepared by: |  |  |  | Ken Jones – 3/24/08 |  |  | Assessment date: |  |  | 3/24/2008 |  |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)