TRAIL ASSESSMENT FORM

| Doc. # | 4021 | Trail Name |  | Great Dome Foot Path North – Chickatawbut Rd. to Sawcut Notch Path | Trail Type: | Narrow - H | # Condition | Good |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route: |  |  | Chickatawbut Road – 4072 – 4073 – 4090 – 4095 - Sawcut Notch Path |  |  |  |  |  |
| Recent Weather: |  |  | Light Rain Today in the 30s |  |  |  |  |  |

| TRAIL LANDMARKS | - Trailhead leaves from the north side of Chickatawbut Road on an unnamed footpath (DCR map) at a granite marker labeled Great Dome Trail. The old FBH map references this trail as the Great Dome Foot Trail. Both maps were used for the assessment. The treadway is footpath width, travels consistently northeast and is lightly blazed primarily with orange rectangles however blue, yellow and green blazes were also noted. (1) - The trailhead is located .16 miles east of the Chickatawbut Overlook and leaves immediately to the right (east) of the granite marker from the east spur path on the Pasture Run trail. - Bn from the to 70 feet. - Bn from 120 through 140 feet. - Cv to the right of the treadway going under Chickatawbut Road. No flow. - Route finding difficult in this area, - Bn at 285 feet. Orange, green and yellow blazes. - Lbd crosses the treadway at 430 feet. 14”. Can be stepped over. - Bn through 500 feet. - Treadway begins ascending at .1 miles. - Treadway crosses the top of this unnamed hill and begins descending at .13 miles. - Bn at .14 miles. Briars encroaching on treadway. - Treadway begins ascending another unnamed hill at .15 miles then on to a large rock outcropping at .17 miles. - Treadway crosses the Ridge Path at intersection marker 4072 at .19 miles. - Bn when re-entering woods as treadway begins descending. - Bn at .22 miles. Yellow blazes noted. - Bn at .24 miles. - Treadway stops descending at .26 miles. - Bn from .27 through .30 miles. - Treadway enters Pine Tree Brook Path at .33 miles and at this point turns left (west) as the Pine Tree Brook and Great Dome Paths share the same treadway to intersection marker 4073. - The Great Dome Foot Path leaves on the right at intersection marker 4073 at .38 miles with orange blazing. - Treadway leaves the height of land and begins descending at .4 miles. - Treadway crosses Pine Tree Brook at .43 miles across a rock dam of small rocks. Lbd prior to crossing can be stepped over 14”. Sbd exiting streambed should be removed 6”. Treadway begins ascending after Pine Tree Brook. - Bn at .46 and .47 miles. - Treadway crosses the height of land at .48 miles with bn. - Sbd crosses at .49 miles. 6”. Cutback. - Sbd crosses at .50 miles. 8”. Cutback. - Bn at .51 and .52 miles. - Sbd crosses at .52 miles. 6”. Cutback. - Bn at .53 and .54 miles. - Treadway enters an intersection with intersection marker 4090 at .57 miles with the Greed Dot and Indian Camp Path. - DCR map shows that the Great Dome Foot Path ends at this intersection. The old FBH map shows that the trail continues northeast following the Green Dot Trail to Crags Footpath at 4095 and then continues over Great Dome to the Sawcut Notch Path. For the purpose of this survey the assessment continues using the FBH map. - Great Dome Foot Path leaves the 4090 intersection on the right (NE) sharing the treadway with the Green Dot Trail. Orange blazes continue. - Bn when re-entering footpath. - Lbd extending into the treadway at .59 miles. Cutback or walk around. - Lbd with bn at .6 miles. Three limbs, two can be dug out and carried off. Remaining 12” can be stepped over. - Bn from .61 through .63 miles. - Bd crosses the treadway at .63 miles. Cut out or step over 12”. - Bn with bd at .64 miles. Cut out or step over 12”. - Bn at .65 miles. - Treadway crosses Crags Foot Path at intersection marker 4095 at .71 miles. Green Dot signage on tree noted. - Treadway reenters the woods (north) at a faded orange blaze at .72 miles and begins ascending a small unnamed hill. - Bn at .73 miles. - Bd crosses the treadway at .74 miles. Cutback. 8” bow saw. - Bn at .75 miles with two bd crossing. 8”. One can be stepped over however the other is waist high and needs to be cutback. Bow saw. - Bn at .76 miles. No blazes with route finding difficult. - Treadway begins descending at .77 miles with significant bn. - Lbd extending into the treadway at .80 miles. Cutback or walk around. - Bn at .81 miles. Route finding difficult. - Treadway begins climbing Great Dome at .87 miles with bn and bd requiring cutback. - Treadway continues to ascend Great Dome just to the right of the large granite slabs. Faded orange and blue blazes. - Bn at .89 miles. - Treadway follows granite slabs at .89 miles with some scrambling required. Faded orange blazes on the slabs. - Treadway crosses the summit of Great Dome at .92 miles with nice views of Chickatawbut and Great Blue Hills from a wonderful lunch rock. - Treadway begins a small descent with orange blazing at .93 miles. - Treadway begins climbing to an additional hill at .96 miles. - Treadway begins descending to Sawcut Notch Path with side bn at .98 miles. - Bn from .99 through 1.0 miles. - Treadway continues its descent on a rock slab to its intersection and ends with the Sawcut Notch Path at 1.03 miles. - Comment: The old FBH map shows the Great Dome Foot Path continuing on to the northeast leaving on the north side of the Sawcut Notch Path. My only indication of this during the survey was a faded orange blaze on a tree 50 yards east of where I entered Sawcut Notch Path at a chain link fence. No treadway was visible. The Granite Links Golf Course now occupies the area beyond the fence. |  |  |  |
| --- | --- | --- | --- | --- |
| MINOR PROBLEM AREAS | - This trail is becoming overgrown in some areas quickly with route finding becoming difficult without foliage. With the growing season upon us and the lack of blazes, this route will become increasingly more difficult to follow. Treadway should be re- blazed ASAP as all blazes are almost faded away. Bn in numerous sections with minor blowdowns as noted in the assessment (1). I’m volunteering for re-blazing, as the trail is a real jewel. Give me the OK and blaze preference? |  |  |  |
| MAJOR PROBLEM AREAS | - None Noted |  |  |  |
| DISTANCES TO MAJOR INTERSECTIONS | - Chickatawbut Road trailhead to Ridge Path at intersection marker 4072 - .19 miles. - Chickatawbut Road trailhead to Pine Tree Brook Path - .33 miles - Chickatawbut Road trailhead to intersection marker 4073 at .38 miles. - Chickatawbut Road trailhead to intersection marker 4090 - .57 miles. - Chickatawbut Road trailhead to Crags Foot Path at intersection marker 4095 - .71 miles. - Chickatawbut Road trailhead to Sawcut Notch Path at 1.03 miles. |  |  |  |
| Report prepared by: |  | Ken Jones 3/31/08 | Assessment date: | 3/29/2008 |

Abbreviations:

Usage type: foot traffic only (H), multi-use (M)

waterbars (wb)               small blowdowns (sbd)

culverts(cv)                     large blowdowns (lbd)

rolling dips (rd)               rock work (rkw)

bridges (br)                     bridge building (bbd),

bog bridges (bb)             culvert replacement (cr)

rock stairs (rs)                 trail relocation (tr),

cribbing (cb)                    building new trail drainage (ntd)

retaining walls (rw)         brushing needed (bn)

ditches (dt)                       excessive erosion (ee)

leaning tree (lt)